| CUT / BOOST SW POS | CS “A” port input active low |  | B_OUTLEVEL_BOOST CS “B” P9 | B_OUTLEVEL_GAIN_EN CS “B” P10 |
| --- | --- | --- | --- | --- |
|  | P0 | P1 |  |  |
| CUT | 0 | 1 | 0 | 1 |
| “OUT” (center / off) | 1 | 1 | X | 0 |
| BOOST | 1 | 0 | 1 | 1 |

| Resolution (gain) SW POS | CS “A” port input active low |  | B_OUTLEVEL_0.25 CS “B” P7 | B_OUTLEVEL_0.5 CS “B” P8 |
| --- | --- | --- | --- | --- |
|  | P14 | P15 |  |  |
| 0 dB | 0 | 1 | 0 | 0 |
| 0.25 dB (center / off) | 1 | 1 | 1 | 0 |
| 0.5 dB | 1 | 0 | 0 | 1 |

| Lift (filters) SW POS | CS “A” port input active low |  | B_FILT_ENABLE (must be on for any filter engaged) CS “C” P12 | B_LOW_FILT_EN CS “C” P13 | B_HI_FILT_EN CS “C” P14 |
| --- | --- | --- | --- | --- | --- |
|  | P9 | P8 |  |  |  |
| 0 (Out) | 1 | 1 | 0 | 0 | 0 |
| 1 (High) | 1 | 0 | 1 | 0 | 1 |
| 2 (Low) | 0 | 1 | 1 | 1 | 0 |
| 3 (Duo / Both high and low) | 0 | 0 | 1 | 1 | 1 |

| Xformer SW POS | CS “A” port input active low |  | B_Xformer_Mode2 CS “B” P11 | XFORMER_TIMED_K1 CS “B” P12 | CS “B” P13 |
| --- | --- | --- | --- | --- | --- |
|  | P7 | P6 |  |  |  |
| 0 (Out) | 1 | 1 | 0 | 1 | 1 |
| 1 (In) | 1 | 0 | 0 | 0 | 0 |
| 2 (In and mode 2) | 0 | 1 | 1 | 0 | 0 |